Dura-ABS™ Direct Injection System Assembly Guide

System Contents:

Dura-ABS™ Pump & Meter System.  1-4 Units

Dura-ABS™ Direct Injection System Frame.  1 unit

Dura-ABS™ DI 2” or 3” Kit.  1-4 units.  (Kit to match Nbr of pumps).

Dura-ABS™ DI 2” or 3” Gauge Kit. 1 unit.

Dura-ABS™ Direct Injection System

Shown with Four (4) ABS pumps, Four (4) – 2” Kits & One (1) 2” Gauge Kit

Dura-ABS™ Direct Injection 2” Kit Assembly		          Dura-ABS™ Direct Injection 2” Gauge Assembly

Dura-ABS™ Direct Injection 3” Kit Assembly	  Dura-ABS™ Direct Injection 3” Gauge Assembly

Dura-ABS™  Direct Injection (DI) 2” Kit.  Contents Include:

- (5) 	DP-T4003raw		1” EPDM Hose					Part A

- (1) 	DP-BAN4001raw		1” Street Elbow – 90-degree				Part B

- (1) 	DP-BANNIP100-4raw	Banjo 1” x 4” Poly Nipple 				Part C

- (1) 	DP-BANNIP100-6raw	Banjo 1” x 6” Poly Nipple				Part D

- (1)	V100SL			Banjo 1” 3-Way Side Load Poly Valve			Part E

- (1)	DP-BAN100Fraw		1” Male Adapter – 1” Male Thread			Part F

- (1)	DP-BAN100125CAP	1” Cap X Male Adapter Poly Cam Lever Coupling		Part G

- (2)    	DP-NF10PHBraw		1” Poly Hose Barb					Part H

- (2)     	DP-HC1000raw		1” Hose Clamp					Part I

- (1)	DP-BANSL100-45raw	Banjo 1” Poly Street Elbow-45 Degree			Part J

- (1)	DP-BANSUV100FPraw	Banjo 1” Full Port Valve				Part K

- (1)	DP-BANM100MPT		1” Flange x 1” Male Thread (NPT)			Part L

- (1)	DP-BANFC100		100 Series 1” Worm Screw Clamp			Part M

- (1)	DP-BANM100G		1” EPDM Manifold Gasket				Part N

- (1)	DP-BANM220100TEE	2” Full Port Flanged Tee x 1” Flanged Tee		Part O

- (1)	DP-BANFC220		220 Series 2” Worm Screw Clamp			Part P

- (1)	DP-BAN200G		220 Series 2” Full Port EPDM Gasket			Part Q

- (4)	DP-ATF 45x1.63x14.5	Torx Screw					Part R

- (1)	Pigtail / Meter-to-Pump	SP-LWA-PM1PM-3D2				Part S

Dura-ABS™  Direct Injection (DI) 2” Gauge Kit.  Contents Include:

- (1) 	DP-BANM220100TEE	2” Full Port Flanged Tee x 1” Flanged Tee		Part OO

- (1) 	DP-BANFC220		220 Series 2” Worm Screw Clamp			Part PP

- (1) 	DP-BAN200G		220 Series 2” Full Port EPDM Gasket			Part QQ

- (1)	DP-BANFC100		100 Series 1” Worm Screw Clamp			 Part MM

- (1)	DP-BANM100G		1” EPDM Manifold Gasket				Part NN

- (1)   	DP-BANM100PLG025	1” Flange plug with ¼” FPT				Part T

- (1)  	DP-P4045 DI-Gauge	Pressure Gauge, 30 PSI Max 2 ½”			Part U

U

T

MM

NN

QQ

PP

OO

G

1

A

Parts identified with “1” from DPEM-4312V Sys.

Parts A thru S = 2” Kit

Double Letters = Gauge Kit

J

O

Q

1

A

C

I

H

E

F

R

D

B

1

1

K

1

1

L

M

N

P

1

1

S

1

A

1

1

1

1” Hose Barb & 1” Hose Clamp at source.

Layout of Dura-ABS™  DI 2” System with 2” Gauge Kit

Assembly Steps for 2” DI System

- Unpack DI Frame from box.

- Assemble Meter Display to frame using the 4 Hex bolts & nuts.

- Insert 2 tube caps on front corners of the frame.

- Place Battery Plate on left rear side of the frame, tighten with 2 Hex nuts.

- Unpack Pump System(s).

NOTE:  Parts from the two Accessory kits in the Pump System box along with parts from the 2” DI kit will be used to assemble entire set-up.

- Using Instruction sheet from the DP-AB001 Accessory kit – (included in the pump box), plumb the pump.

- Place pump into the DI frame, secure using 3 snap pins (1).  Two in front, one in the rear.

- Attach (B) to outlet of the meter.

- Attach either (C) or (D) to (B).

  - For position 1 and 2, use (C).

  - For position 3 and 4, use (D).

- Attach 1” Elbow Hose Barb (1) to the inlet of the meter.

- Attach meter to the Meter Display using 4 Torx screws (R).

- Insert (S) between pump power cord and the meter power cord.

- Attach (E) to (C) or (D).

- Attach (F) to the top of (E).

- Place (G) on top of (F).

- Insert (H) into bottom of the (E).

- Attach 1” hose from inlet of meter to outlet of pump.  Tighten with 1” Hose Clamp.  Can cut hose to length.

- Starting from Manifold

  - Attach (O) to (L) using parts (M) & (N).

  - Attach 1” Check Valve (1), make sure arrow on check valve points toward (O).

  - Attach 1” Poly Nipple (1) to Check Valve (1).

  - Attach Ball Valve (K).

  - Attach (J) to (K).

  - Attach 1” Hose barb (1) to (J).

  - Attach 1” hose from step 17 to the 1” hose barb (1) & tighten with 1” hose clamp (1).

- To attach Gauge kit to Manifold

  - Attach (O) to (OO) using parts (P) & (Q).

  - Attach (T) to (OO) using parts (MM) & (NN).

  - Attach (U) to (T).

  - Attach (PP) and (QQ) to outlet of (OO).

- Attach 1” hose from bottom of (E) to manifold.

- Attach long power cord (I) from meter to battery.

Dura-ABS™  Direct Injection (DI) 3” Kit.  Contents Include:

- (5) 	DP-T4003raw		1” EPDM Hose					Part A

- (1) 	DP-BAN4001raw		1” Street Elbow – 90-degree				Part B

- (1) 	DP-BANNIP100-4raw	Banjo 1” x 4” Poly Nipple 				Part C

- (1) 	DP-BANNIP100-6raw	Banjo 1” x 6” Poly Nipple				Part D

- (1)	V100SL			Banjo 1” 3-Way Side Load Poly Valve			Part E

- (1)	DP-BAN100Fraw		1” Male Adapter – 1” Male Thread			Part F

- (1)	DP-BAN100125CAP	1” Cap X Male Adapter Poly Cam Lever Coupling		Part G

- (2)    	DP-NF10PHBraw		1” Poly Hose Barb					Part H

- (2)     	DP-HC1000raw		1” Hose Clamp					Part I

- (1)	DP-BANM300TEE		3” Flanged Tee					Part J

- (1)	DP-BANFC300		300 Series Worm Screw Clamp			Part K

- (1)	DP-BAN300G		300 Series EPDM Gasket				Part L

- (1)	DP-BANM300PLG100	3” Flange Plug with 1” FPT				Part M

- (1)	DP-BANSUV100FPraw	Banjo 1” Full Port Valve				Part O

- (4)	DP-ATF 45x1.63x14.5	Torx Screw					Part P

- (1)	Pigtail / Meter-to-Pump	SP-LWA-PM1PM-3D2				Part Q

Dura-ABS™  Direct Injection (DI) 3” Gauge Kit.  Contents Include:

- (1) 	DP-BANM300TEE		3” Flanged Tee					Part JJ

- (2) 	DP-BANFC300		300 Series Worm Screw Clamp			Part KK

- (2) 	DP-BAN300G		300 Series EPDM Gasket				Part LL

- (1)	DP-BANM300PLG075	3” Flange Plug with 3/4” FPT				Part R

- (1)     	DP-BANRB075-025raw	3/4” MPT x ¼” FPT Reducing Bushing			Part S

- (1)	DP-P4045 DI-Gauge	Pressure Gauge, 30 PSI Max 2 ½”			Part T

R

T

S

LL

KK

A

1” Hose Barb & 1” Hose Clamp at source.

KK

LL

JJ

K

L

O

1

1

M

L

K

J

I

H

1

I

H

G

F

E

D

C

B

P

AQ

A

Q

1

1

1

1

1

1

1

1

Double Letters = Gauge Kit

Parts A thru S = 3” Kit

Parts identified with “1” from DPEM-4312V Sys.

Parts identified with “1” from DPEM-4312V Sys.

Layout of Dura-ABS™  DI 3” System with 3” Gauge Kit

Assembly Steps for 3” DI System

- Unpack DI Frame from box.

- Assemble Meter Display to frame using the 4 Hex bolts & nuts.

- Insert 2 tube caps on front corners of the frame.

- Place Battery Plate on left rear side of the frame, tighten with 2 Hex nuts.

- Unpack Pump System(s).

NOTE:  Parts from the two Accessory kits in the Pump System box along with parts from the 3” DI kit will be used to assemble entire set-up.

- Using Instruction sheet from the DP-AB001 Accessory kit – (included in the pump box), plumb the pump.

- Place pump into the DI frame, secure using 3 snap pins (1).  Two in front, one in the rear.

- Attach (B) to outlet of the meter.

- Attach either (C) or (D) to (B).

  - For position 1 and 2, use (C).

  - For position 3 and 4, use (D).

- Attach 1” Elbow Hose Barb (1) to the inlet of the meter.

- Attach meter to the Meter Display using 4 Torx screws (P).

- Insert (Q) between pump power cord and the meter power cord.

- Attach (E) to the (C) or (D).

- Attach (F) to the top of (E).

- Place (G) on top of (F).

- Insert (H) into bottom of the (E).

- Attach 1” hose from inlet of meter to outlet of pump.  Tighten with 1” Hose Clamp.  Can cut hose to length.

- Starting from Manifold

  - Attach (J) to (M) using parts (K) & (L).

  - Attach 1” Poly Nipple to (M).

  - Attach 1” Check Valve (1), make sure arrow on Check Valve points toward (J).

  - Attach 1” Poly Nipple (1) to Check Valve (1), attach Ball Valve (O).

  - Attach 1” Hose barb (1) to Ball Valve (O).

  - Attach 1” hose from step 17 to the 1” hose barb (1) & tighten with 1” hose clamp (1).

- To attach Gauge kit to Manifold

  - Attach (J) to (JJ) using parts (K) & (L).

  - Attach (JJ) TO (R) using parts (KK) & (LL).

  - Attach (S) to (R).

  - Attach (T) to (S).

- Attach 1” hose from bottom of (E) to manifold.

- Attach long power cord (I) from meter to battery.